Terminology:

- Access Console System – System you will be remoting from.

- Jump System – System you will be remoting to.

Requirements:

- UCSF MyAccess Account

- Access Console System

  - Windows, Mac, Linux, iOS, and Android System

  - Administrator Rights

  - Network Access to – remotesupport.ucsf.edu on Port 443

- Jump Client System (Target System)

  - Windows, Mac or Linux

  - Administrator Rights

  - Network Access to – remotesupport.ucsf.edu on Port 443

Request Access:

- Submit a Ticket to ITFS Desktop Engineering to grant access and provide the following information:

  - Users Email Address - MyAccess

  - Type of Remote Access

    - Remote Access – Screen, Keyboard and Mouse control

    - Remote Access via Jumpoint (Jump Server) – Advanced Users

  - Login Schedule: Any restrictions for users to login and use Remote Access

  - Invite Collaborators: Yes or No

Installing Jump Client on Jump Systems:

- Once ITFS Desktop Engineering grant you access, you will need to create installers.

  - Login to https://remoteaccess.ucsf.edu/login

    - Click “Use SAML Authentication”

  - Go the “Jump” Tab, and make sure the drop down options match the screenshot, then click “Create”

    - ***If you have a Team that needs access to the Jump System, select your Jump Group, before you click “Create” (ie. UCSF – Dept)

  - Select the Platform you will install the jump client on and click “Download”.  **For Macs, we recommend “macOS (for Manual Execution)

  - Copy the client to the Jump System and install.

  - For Windows:

    - When installing, it will show

    - Once it goes away, the client is installed.  If you want to verify, you can go to Programs and Features in Control Panel

  - For Macs:

    - When installing, it will ask if it is ok to install, go ahead and proceed.

    - You will be prompted to open “System Preferences”, click “Open System Preferences”

    - Check and allow PRA.

      - It will prompt to restart the app, click “Quit & Reopen”

      - The jump client install is **NOT** complete.  You will need to install the Access Console with the instructions below.

      - Once you have the Access Console installed, connect to the system and it will ask to configure additional security controls.

        - Make sure you click “Grant Access” to Accessibility and Full Disk Access”

      - After this the Jump Client install is now complete.

Installing Access Console:

- Windows, Mac, and Linux

  - Login to remoteaccess.ucsf.edu/login

  - Under “My Account” and “Access Console”.  Select your platform and click “Download Access Console” and run the installer

- Android and iOS

  - Apple App Store and Google Play Store

    - Search for “BeyondTrust Access Console”

- Web Console

  - https://remoteaccess.ucsf.edu/console

Access Console:

- Run the BeyondTrust Privileged Remote Access Console

- Make sure to select “SAML Credentials” and click “Login”

- A web browser will open and prompt you for your MyAccess Login

- Once you login, it will download a token, go ahead and open the token.

- The application will open, and you should see the console

- If you install the jump client, under “My Jump Group”, click “Personal” and you should see the system.

- To remote to the system, double click the system.

- You will see the system screen and be able to login to it.

Access Console Guides:

- Access Console – Windows, Mac and Linux

  - https://www.beyondtrust.com/docs/privileged-remote-access/getting-started/access-console/index.htm

- Access Console – Web Console

  - https://www.beyondtrust.com/docs/privileged-remote-access/getting-started/access-console/web-access/index.htm

- Access Console – iOS

  - https://www.beyondtrust.com/docs/privileged-remote-access/getting-started/access-console/apple-ios/index.htm

- Access Console – Android

  - https://www.beyondtrust.com/docs/privileged-remote-access/getting-started/access-console/android/index.htm

Links:

- https://www.beyondtrust.com/docs/privileged-remote-access/index.htm

Notes:

- If a Jump Client goes offline and does not reconnect to the server for 180 days, it is automatically uninstalled from the target computer and is removed from the Jump interface.